# Internet

by Spencer Tiberi

## Introduction

- We use the internet on a daily basis and have constant access and connectivity

- Home network

  - Cable modem, DSL modem, or FIOS device

    - Connects to the internet

    - Pay monthly for an ISP (Internet Service Provider)

      - Verizon, Comcast, etc.

    - Could have built in wireless connectivity for your devices

      - May need an additional home router

        - Devices connect to a router via cables or wifi

## IP

- Every computer on the internet has an IP (Internet Protocol) address

  - Of the form #.#.#.#

    - Four numbers separated by dots of the values 0-255

    - Other IP address formats exist today as well

  - Like postal addresses, they uniquely identify computers on the internet

    - Any device connected to the internet has an IP address

      - Allows other computers to talk to it

- ISPs assign a IP address to your computer (router)

  - Used to be physically configured

  - DHCP (Dynamic Host Configuration Protocol)

    - Software that ISPs provides to allow your computer to request an IP address

    - DHCP servers respond with a specific IP address for your Home

  - Multiple devices can connect to your home network

    - The home router supports DHCP and assigns IP addresses to your devices

## DNS

- We access websites using domain names (Facebook.com, Google.com, etc.), but it turns out that these sites too have IP addresses

- DNS (Domain Name System) servers convert domain names into IP addresses

## Packets

- Computers communicate by sending packets, which are like virtual envelopes sent between computers

  - Ultimately still 0s and 1s

- As an analogy, suppose we want to find a cat image on the internet

- So, we send a request to a server, say Google, like “get cat.jpg”

  - We place this request in an envelope

- On the envelope, we list out IP as the return address

- However, for the recipient of the request, we don’t know the IP address for Google

  - Have to rely on DNS

  - Send a request to our ISPs DNS server for Google’s IP address

    - If the ISP’s DNS server doesn’t know a website’s IP address, it has been configured to ask another DNS server

    - There exist root servers that know where to look to for an IP address if it exists

- After sending the request off, we’ll get a response ms later

- The cat will be sent back in one or more packets

  - If the cat image is too large for a single envelope, sending it in one packet could take up internet traffic

  - To solve this, Google will divide the cat image into smaller fragments

    - Put the fragments into different envelopes

    - Write information on the envelopes

      - Return address: Google’s IP address

      - Delivery address: Our IP address

      - List the number of packets on each envelope (1 of 4, 2 of 4, etc.)

## TCP/IP

- IP goes beyond addresses

  - Set of conventions computers and servers follow to allow intercommunication

- Fragmentation like in the envelope example are supported by IP

  - If missing a packet, you can logically infer which packet you’re missing based on the ones received

    - However, IP doesn’t tell computers what to do in this case

- TCP (Transmission Control Protocol) ensures packets can get to their destination

  - Commonly used with IP (TCP/IP)

  - Supports sequence numbers that help data get to its destination

    - When missing a packet, a computer can make a request for the missing packet

    - The computer will put packets together to get a whole file

  - Also includes conventions for requesting services (port identifiers)

    - To make sure Google knows we’re requesting a webpage and not an email or other service

## Ports

- Per TCP, the world has standardized numbers that represent different services

- If 5.6.7.8 is Google’s IP address, 5.6.7.8;80 (port 80) lets use know that we want a webpage

  - 80 means http (hypertext transfer protocol)

    - The language that web servers speak

  - Google will send the request to their web server via http

- Many websites use secure connections with SSL or HTTPS, which uses the port 443

- Email uses port 25

- Other ports exist as well

## Protocols

- Protocols are just sets of rules

  - Humans use these all the time, such as the protocol for meeting people: handshakes

- When a request is made to Google for an image, HTTP tells Google how to respond appropriately

## UDP

- User Datagram Protocol

  - Doesn’t guarantee delivery

  - Used for video conferencing such as FaceTime

    - Packets can be dropped for the sake of keeping the conversation flowing

  - Used anytime you want to keep data coming without waiting for a buffer to fill

## IPs in More Detail

- IP addresses are limited

  - In the format #.#.#.#, each number is 8 bits, so 32 bits total

    - This yields 232 or about 4 billion possible addresses

      - We’re running out of addresses for all computers

  - Current version of addresses is IPv4

  - Moving towards IPv6

    - Uses 128 bits, yielding 2128 possible addresses

- How do you find your IP address?

- On a Mac, go to system preferences an poke around a bit

- Private addresses exist

  - 10.#.#.#, 192.168.#.#, or 172.16.#.#

  - Only with special configuration can someone talk to your computer

  - Your personal device is not a server, so people should not need to access them directly

    - Your device needs to request data from servers

  - Even email is stored on a server such as Gmail and your device makes a request to that server to access that email

- Looking at advanced settings…

  - Subnet mask is used to decide if another computer is on the same network

  - Router (aka Gateway) has its own address

    - Routs data in different directions

- On windows:

  - Shows DNS servers as well

## Routers

- Routers have bunches if wires coming and going out of them

  - They have a big table with IP addresses and where data should be routed to get to that destination

    - Often, the data is routed to some next router

- Routers purpose is to send data in the direction of a destination

  - The next router will send it to another until it reaches a destination

- The internet is a network of networks (with their own routers)

  - Often multiple ways to go from A to B

    - Based in US Military logic to prevent downtime if a particular router goes down

    - When multiple packets are sent, like cat.jpg from Google, they can each take a different path, still getting to their destination eventually

      - Sometimes the internet is busy and the quickest path changes

## Traceroute

- How long does it take for this process of data transfer to take on the internet?

- Traceroute is a program that sends packets to each router on a path to a destination, reporting the time it takes to reach that router

- From Sanders Theatre to Google.com:

  - 1-2: A few unnamed routers at Harvard

  - 3-4: More Harvard routers

  - 5-6: Level3 is a ISP

  - 7+: The routers are denying the request

- From Sanders Theatre to Berkeley.edu

  - 6: Northern Crossroads

  - 7-14: A fast connection

    - 8-9: Chicago

    - 10-11: Denver

    - 12-13: Las Vegas

    - 14: Los Angeles

  - 19 is where it arrives at Berkeley in 80 ms!

- From Sanders Theatre to MIT.edu

  - 6-7: Goes to New York connectivity

  - 8: MIT’s website is outsourced to Akamai’s NYC servers

- From Sanders Theatre to CNN.jp

  - 9-10 jumps from Seattle to Osaka past an ocean!

    - Using undersea cabling

## Undersea Cabling

- David shows a video about undersea cables

## Cable Modem Demo

- David examines a home cable modem, focusing on its ports

  - Coaxial cable to plug into the wall

  - Phone jacks (RJ11) as many services are bundled together these days

  - Four jacks for ethernet cables (RJ45)

    - Devices can plug into these for internet connectivity

  - This modem has wifi support built in

## Network Switch Demo

- David examines a network switch

  - A device that you can plug into your router to allow more connections for all your other devices

## Home Router Demo

- David examines a home router

- Home routers can have wifi, firewall, and switching capabilities

## Network Cable Demo

- David cuts open a network ethernet cable to examine its inner workings

- Inside a network cable are 8 wires of different colors

  - Some are for transmitting data, others for receiving data

  - Others still are for insulation and cancellation of interference

## Closing Thoughts and Homework

- For homework, find a device that looks like a modem or router and take a look at the connectors on the back of it

  - If brave, play around with unplugging cables

    - Note: Your internet may go down in the process, but can be easily restarted with the cables properly reconnected!

  - If you have a spare ethernet cable, take a look inside yourself

    - These are a bit harder to put back together!